# CasScale.OCX

Version (2.3.5.0)

- Introduction of CasScale.DLL

- Function Explanation

- Data Structure

- Define

April, 2009

CAS Corporation

1. Introduction of CasScale.OCX

CasScale.OCX can control CAS label printing scale effectively which are networked together with systems. CasSale.OCX consists of many functions which can be used in developing application programs easily for the interface between system software and CAS label printing scales.

CAS CasScale.OCX consists of the followings and they should be applied to the application program as one package,

CASSCALE.OCX

CASPRTC.DLL

CASTCPIP.DLL

CASSERIAL.DLL

CLinterpreter.DLL

CLJRInterpreter.DLL

LPRUSInterpreter.DLL

APPInterpreter.DLL

FileTrans.DLL

DATAOPTION.INI

DATAOPTION_RUS.INI

The functions are listed as follows (ordered by alphabetical);

Connection(IP,PORT,SCALEID,MODEL)

ConnectionEx2(IP,Port,ScaleID,Model,Version,DataType)  // New (Recommand)

AddPLU(PLUDATA)

AddMessage(MESSAGEDATA)

SendPLU()

SendData(IP, Model, Version, TransType, DataType, Data)  //New (Recommand)

SendMSG()

RecvPLU()

RecvMSG()

RecvPLUData(PLUDATA)

RecvMSGData(MESSAGEDATA)

DeletePLU(DEPARTMENTNO,PLUNO)

DeleteAllPLU(DEPARTMENTNO,PLUNO)

DeleteMSG(MESSAGENO)

GetScaleInfo()

GetScaleInfoData(IP,INFO);

GetState()

GetTransStatus(IP,TYPE,STATUS)

GetLibVersion()

DisconnectAll();

ReadPLU(DEPARTNO,PLUNO)

ReadMSG(MESSAGENO)

Stop()

2. Function Explanation

2.1 long Connection(IP,PORT,SCALEID,MODEL)

Description: Register scales already connected to network.

Parameter :

IP: String, IP of scale already registered, ex) “192.168.1.1”

PORT: integer, Port Number of separate scale

If ‘0’, configure that value automatically according to ‘MODEL’.

If MODEL=5000, PORT = 20304

If MODEL=161,  PORT = 8111

If MODEL=5010, PORT = 20304

SCALEID: integer, SCALEID of scale

In the case of LP-1.6 & LP-II, this value should not be ‘0’.

This value should be same as SCALEID.

MODEL: integer

Model=161,  LP-1.6 & LP-II

Model=5000, CL5000

Model=5010, CL5000Jr

Return    : long

0 = Not registered. (Scale is not connected to the network.)

1 = Scale registered Normally

2 = Scale registered Already

Reference : DisconnectAll()

2.1.1 long ConnectionEx2(IP,PORT,SCALEID,MODEL,VERSION,DATATYPE)

Description: Register scales already connected to network.

If you want to use SendData(), you must connect to scale with ConnectionEX2()

Parameter :

IP: String, IP of scale already registered, ex) “192.168.1.1”

PORT: integer, Port Number of separate scale

If ‘0’, configure that value automatically according to ‘MODEL’.

If MODEL=5000, PORT = 20304

If MODEL=161,  PORT = 8111

If MODEL=5010, PORT = 20304

SCALEID: integer, SCALEID of scale

In the case of LP-1.6 & LP-II, this value should not be ‘0’.

This value should be same as SCALEID.

MODEL: integer

Model=161,  LP-1.6 & LP-II

Model=5000, CL5000

Model=5010, CL5000Jr

VERSION : String

It is a verion of scale

You can take this verion with SendData().

Refer the SendData()

DATATYPE : integer

It is a type of data which you want to send

Ex) PLU : DF_DATA_PLU_V02

Refer “5.Define” part

Return    : long

0 = Not registered. (Scale is not connected to the network.)

1 = Scale registered Normally

Reference : DisconnectAll()

2.2 long AddPLU(PLUDATA)

Description: Register PLU for transmission.

Parameter :

PLUDATA: String

Make PLU Data according to the Table 4.1.

Ex 1) Model=161 & 5000

"000100000101000000003345000000000000026120000000000001000000000000080000000000000000000000000000001000000000000000000000000000000000000000000000000000000000000НТ64    Название1                                             Название2                                             Код Рост"

Ex 2) Model=5010

"000100000101000000003345000000000000026120000000000001000000000000080000000000000000000000000000001000000000000000000000000000000000000000000000000000000000000НТ64    Название1                                             Название2                                             Код Рост                    Прямое сообщение”

Return    : long,  Return the number of registered PLUs.

Reference : SendPLU()

Reference: You can use Ex 1 and 2. You don’t need to make data full on a last string data but you should follow up the length of digits on a last variable data. And you can not transmit the direct ingredient to the scale that doesn’t support direct ingredient.

2.3 long AddMSG(MESSAGEDATA)

Description: Register message for transmission.

Parameter :

MESSAGEDATA: String

Make Message Data according to the Table of 4.2.

Ex) “000001MessageAz012345678901234567890123456789012345678901234Message1 Line2”

Return    : long, Return the number of registered MESSAGEs

Reference : SendMSG()

2.4 long SendPLU()

Description: Send all PLUs registered through AddPLU() to all scales.

Parameter : NONE

Return    : long, If not ‘0’, start to send.

Reference : AddPLU(), GetState()

To use CAS communication module, all of the following libraries & ‘DATAOPTION.ini’ should be applied to the application program.

CASPRTC.DLL

CASTCPIP.DLL

CLinterpreter.DLL

CLJRInterpreter.DLL

LPRUSInterpreter.DLL

FileTrans.DLL

DATAOPTION.INI

DATAOPTION.INI is a defined file for communication with CL5000. Through this you can change data size variably.

The files above are recommended to be located in the same path of the application program. If the files are located in Windows path (example; C://Windows//system32), the application program and ‘DATAOPTION.ini’ should be located in the same path.

PLU field selection: PLU with selected fields can be downloaded to the scale thru ‘SendPLU’ interface depending on programmer’s selection. Downloading PLU with selected fields is faster than downloading PLU with all fields. So, It is recommended in most application to use downloading PLU with selected fields which are practically used in application..

CasScale.OCX consists of 34 PLU fields and The PLU fields are located in ‘DATAOPTION.ini’ as Meta file.  LEVEL of PLU field should be defined on “PLU_FIELD _LEVEL = X’ (X should be 1, 2, 3…8 or 9 ) in the ‘DATAOPTION.ini’ depending on the application for efficiency.  If ‘LEVEL = 1’, the field group of ‘LEVEL=1’ is downloaded, if ‘LEVEL=2’, the field group of ‘LEVEL=1’ & ‘LEVEL=2’ are downloaded.  If ’LEVEL=3’,  the field group of ‘LEVEL=1’, ‘LEVEL=2’ and ‘LEVEL=3’ are downloaded.  Please refer to the following example.

Example)

[FILE_INFO]

FILE_VER = 1

FILE_USE = 1

COUNTRY_NAME = RU

KEY_COUNT = 1

KEY_LIST = PLU_STRUCTURE,

MODEL_COUNT = 1

MODEL_LIST = 5000,

[SUBSTRUCTURE_INFO]

SUBSTRUCTURE_USE = 0

SUBSTRUCTURE_COUNT = 0

SUBSTRUCTURE_ITEM1_MODEL =

SUBSTRUCTURE_ITEM1_FILE =

[PLU_STRUCTURE_5000]

PLU_STRUCTURE_VER = 1

PLU_STRUCTURE_USE = X    (X should be 1, 2, 3…8 or 9)

PLU_STRUCTURE_CODE = 1100

PLU_STRUCTURE_ITEM_COUNT = 48

PLU_FIELD1_CODE = 1

PLU_FIELD1_NAME = Department No.

PLU_FIELD1_SIZE = 2

PLU_FIELD1_LENG = 2

PLU_FIELD1_MIN  = 1

PLU_FIELD1_MAX  = 99

PLU_FIELD1_DEC  = 0

PLU_FIELD1_LEVEL= 1

PLU_FIELD1_DEFAULT = 1

PLU_FIELD1_DATATYPE = 10

PLU_FIELD1_DATACLAS = 2

PLU_FIELD7_CODE = 10

PLU_FIELD7_NAME = Name

PLU_FIELD7_SIZE = 100

PLU_FIELD7_LENG = 40

PLU_FIELD7_MIN  = 0

PLU_FIELD7_MAX  = 0

PLU_FIELD7_DEC  = 0

PLU_FIELD7_LEVEL= 1

PLU_FIELD7_DEFAULT = 1

PLU_FIELD7_DATATYPE = 1

PLU_FIELD7_DATACLAS = 1

PLU_FIELD13_CODE = 16

PLU_FIELD13_NAME = Sell by date

PLU_FIELD13_SIZE = 2

PLU_FIELD13_LENG = 4

PLU_FIELD13_MIN  = 0

PLU_FIELD13_MAX  = 9999

PLU_FIELD13_DEC  = 0

PLU_FIELD13_LEVEL= 2

PLU_FIELD13_DEFAULT = 0

PLU_FIELD13_DATATYPE = 10

PLU_FIELD13_DATACLAS = 2

PLU_FIELD16_CODE = 19

PLU_FIELD16_NAME = Packed Time

PLU_FIELD16_SIZE = 2

PLU_FIELD16_LENG = 2

PLU_FIELD16_MIN  = 0

PLU_FIELD16_MAX  = 99

PLU_FIELD16_DEC  = 0

PLU_FIELD16_LEVEL= 3

PLU_FIELD16_DEFAULT = 0

PLU_FIELD16_DATATYPE = 10

PLU_FIELD16_DATACLAS = 2

In case LEVELS are defined as above;

- PLU_STRUCTURE_USE = 1

‘Department No.’ & ‘Name’ in the ‘LEVEL = 1’ will be downloaded.

- PLU_STRUCTURE_USE = 2

The fields(‘Department No.’ & ‘Name’) in ‘LEVEL=1’ including the fields(‘Sell by date’) in ‘LEVEL=2’ will be downloaded.

- PLU_STRUCTURE_USE = 3

The field(‘Packed time’) in ‘LEVEL=3’ including ‘LEVEL=1’ & ‘LEVEL=2’ will be downloaded.

[Initial setting of LEVEL of PLU fields in ‘DATAOPTION.ini’]

| No. | Field |  | LEVEL |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | In ‘DATAOPTION’ | In Russian | 1 | 2 | 3 | 9 |
| 1 | Department No | Отдел № | + |  |  |  |
| 2 | PLU No | Товар № | + |  |  |  |
| 3 | PLU Type | Тип товара | + |  |  |  |
| 4 | Unit weight | Масса для цены |  |  |  | + |
| 5 | Price | Цена | + |  |  |  |
| 6 | Group No. | Группа № | + |  |  |  |
| 7 | Itemcode | Код товара | + |  |  |  |
| 8 | Tare No. | Тара № |  |  |  | + |
| 9 | Tare | Масса тары | + |  |  |  |
| 10 | Pieces | Кол-во товаров в наборе |  |  |  | + |
| 11 | Q’ty unit No. | Наименов. единицы № |  |  | + |  |
| 12 | Sell by date | Годен до (дата) | + |  |  |  |
| 13 | Sell by time | Годен до (дата) |  |  | + |  |
| 14 | Packed date | Дата упаковки | + |  |  |  |
| 15 | Packed time | Время упаковки |  | + |  |  |
| 16 | Produce date | Дата производства |  |  | + |  |
| 17 | Ingredient No. = MSG | Состав продукта № |  |  | + |  |
| 18 | Use fixed price type | Фиксиров. Стоимость |  |  | + |  |
| 19 | Traceability No. | Перемещ. мясопрод. № |  |  |  | + |
| 20 | Origin No. | Страна происх. № |  |  |  | + |
| 21 | Nutrifact No. | Пищевая ценность № |  |  |  | + |
| 22 | Label No. | Формат этикетки № |  |  | + |  |
| 23 | Aux Label No. | Доп. формат № |  |  |  | + |
| 24 | Barcode No. | Формат штрих-кода № |  |  | + |  |
| 25 | Barcode2 No. | Формат доп. штрих-кода № |  |  |  | + |
| 26 | Sales Msg No. | Доп. текст № |  |  | + |  |
| 27 | SpecialPrice | Специальная цена |  |  |  | + |
| 28 | Fixed weight | Заданная масса |  |  |  | + |
| 29 | Logo No | Логотип № |  |  |  |  |
| 30 | Logo Name | Наимен. Логотипа (Ростест №) |  |  |  |  |
| 37 | Name | Наимен. 1 | + |  |  |  |
| 32 | Name2 | Наимен. 2 | + |  |  |  |
| 33 | Name3 (Rostest code) * | Код Ростеста * |  | + |  |  |
| 34 | DirectIngredient |  | + |  |  |  |

*  Rostest field for CL5000 & CL5000Jr

**  Rostest field for LP 1.6 & LP-II

As above LEVELs of PLU fields are defined as default in the ‘DATAOPTION.ini’.  You can change LEVELs on your selection.  The fields of ‘LEVEL = 9’ are disabled  in the current CL5000 version but reserved for future use.

Reference : Direct ingredient exists on the file, but level setting value is not effective.

If a direct ingredient exists, it sends that. Else, it doesn’t send that.

That is ignored when it communicates with a scale that doesn’t support a direct ingredient

2.4.1 long SendData()

Description: Send one PLU. You don’t have to register PLU before using SendData().

It means that You don’t have to use AddPLU()

Parameter : NONE

Return    : long, If not ‘0’, start to send.

To use CAS communication module, all of the following libraries & ‘DATAOPTION.ini’ should be applied to the application program.

CASPRTC.DLL

CASTCPIP.DLL

CLinterpreter.DLL

CLJRInterpreter.DLL

LPRUSInterpreter.DLL

FileTrans.DLL

DATAOPTION.INI

DATAOPTION.INI is a defined file for communication with CL5000. Through this you can change data size variably.

The files above are recommended to be located in the same path of the application program. If the files are located in Windows path (example; C://Windows//system32), the application program and ‘DATAOPTION.ini’ should be located in the same path.

PLU field selection: PLU with selected fields can be downloaded to the scale thru ‘SendPLU’ interface depending on programmer’s selection. Downloading PLU with selected fields is faster than downloading PLU with all fields. So, It is recommended in most application to use downloading PLU with selected fields which are practically used in application..

CasScale.OCX consists of 34 PLU fields and The PLU fields are located in ‘DATAOPTION.ini’ as Meta file.  LEVEL of PLU field should be defined on “PLU_FIELD _LEVEL = X’ (X should be 1, 2, 3…8 or 9 ) in the ‘DATAOPTION.ini’ depending on the application for efficiency.  If ‘LEVEL = 1’, the field group of ‘LEVEL=1’ is downloaded, if ‘LEVEL=2’, the field group of ‘LEVEL=1’ & ‘LEVEL=2’ are downloaded.  If ’LEVEL=3’,  the field group of ‘LEVEL=1’, ‘LEVEL=2’ and ‘LEVEL=3’ are downloaded.  Please refer to the following example.

Example)

[FILE_INFO]

FILE_VER = 1

FILE_USE = 1

COUNTRY_NAME = RU

KEY_COUNT = 1

KEY_LIST = PLU_STRUCTURE,

MODEL_COUNT = 1

MODEL_LIST = 5000,

[SUBSTRUCTURE_INFO]

SUBSTRUCTURE_USE = 0

SUBSTRUCTURE_COUNT = 0

SUBSTRUCTURE_ITEM1_MODEL =

SUBSTRUCTURE_ITEM1_FILE =

[PLU_STRUCTURE_5000]

PLU_STRUCTURE_VER = 1

PLU_STRUCTURE_USE = X    (X should be 1, 2, 3…8 or 9)

PLU_STRUCTURE_CODE = 1100

PLU_STRUCTURE_ITEM_COUNT = 48

PLU_FIELD1_CODE = 1

PLU_FIELD1_NAME = Department No.

PLU_FIELD1_SIZE = 2

PLU_FIELD1_LENG = 2

PLU_FIELD1_MIN  = 1

PLU_FIELD1_MAX  = 99

PLU_FIELD1_DEC  = 0

PLU_FIELD1_LEVEL= 1

PLU_FIELD1_DEFAULT = 1

PLU_FIELD1_DATATYPE = 10

PLU_FIELD1_DATACLAS = 2

PLU_FIELD7_CODE = 10

PLU_FIELD7_NAME = Name

PLU_FIELD7_SIZE = 100

PLU_FIELD7_LENG = 40

PLU_FIELD7_MIN  = 0

PLU_FIELD7_MAX  = 0

PLU_FIELD7_DEC  = 0

PLU_FIELD7_LEVEL= 1

PLU_FIELD7_DEFAULT = 1

PLU_FIELD7_DATATYPE = 1

PLU_FIELD7_DATACLAS = 1

PLU_FIELD13_CODE = 16

PLU_FIELD13_NAME = Sell by date

PLU_FIELD13_SIZE = 2

PLU_FIELD13_LENG = 4

PLU_FIELD13_MIN  = 0

PLU_FIELD13_MAX  = 9999

PLU_FIELD13_DEC  = 0

PLU_FIELD13_LEVEL= 2

PLU_FIELD13_DEFAULT = 0

PLU_FIELD13_DATATYPE = 10

PLU_FIELD13_DATACLAS = 2

PLU_FIELD16_CODE = 19

PLU_FIELD16_NAME = Packed Time

PLU_FIELD16_SIZE = 2

PLU_FIELD16_LENG = 2

PLU_FIELD16_MIN  = 0

PLU_FIELD16_MAX  = 99

PLU_FIELD16_DEC  = 0

PLU_FIELD16_LEVEL= 3

PLU_FIELD16_DEFAULT = 0

PLU_FIELD16_DATATYPE = 10

PLU_FIELD16_DATACLAS = 2

In case LEVELS are defined as above;

- PLU_STRUCTURE_USE = 1

‘Department No.’ & ‘Name’ in the ‘LEVEL = 1’ will be downloaded.

- PLU_STRUCTURE_USE = 2

The fields(‘Department No.’ & ‘Name’) in ‘LEVEL=1’ including the fields(‘Sell by date’) in ‘LEVEL=2’ will be downloaded.

- PLU_STRUCTURE_USE = 3

The field(‘Packed time’) in ‘LEVEL=3’ including ‘LEVEL=1’ & ‘LEVEL=2’ will be downloaded.

[Initial setting of LEVEL of PLU fields in ‘DATAOPTION.ini’]

| No. | Field |  | LEVEL |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | In ‘DATAOPTION’ | In Russian | 1 | 2 | 3 | 9 |
| 1 | Department No | Отдел № | + |  |  |  |
| 2 | PLU No | Товар № | + |  |  |  |
| 3 | PLU Type | Тип товара | + |  |  |  |
| 4 | Unit weight | Масса для цены |  |  |  | + |
| 5 | Price | Цена | + |  |  |  |
| 6 | Group No. | Группа № | + |  |  |  |
| 7 | Itemcode | Код товара | + |  |  |  |
| 8 | Tare No. | Тара № |  |  |  | + |
| 9 | Tare | Масса тары | + |  |  |  |
| 10 | Pieces | Кол-во товаров в наборе |  |  |  | + |
| 11 | Q’ty unit No. | Наименов. единицы № |  |  | + |  |
| 12 | Sell by date | Годен до (дата) | + |  |  |  |
| 13 | Sell by time | Годен до (дата) |  |  | + |  |
| 14 | Packed date | Дата упаковки | + |  |  |  |
| 15 | Packed time | Время упаковки |  | + |  |  |
| 16 | Produce date | Дата производства |  |  | + |  |
| 17 | Ingredient No. = MSG | Состав продукта № |  |  | + |  |
| 18 | Use fixed price type | Фиксиров. Стоимость |  |  | + |  |
| 19 | Traceability No. | Перемещ. мясопрод. № |  |  |  | + |
| 20 | Origin No. | Страна происх. № |  |  |  | + |
| 21 | Nutrifact No. | Пищевая ценность № |  |  |  | + |
| 22 | Label No. | Формат этикетки № |  |  | + |  |
| 23 | Aux Label No. | Доп. формат № |  |  |  | + |
| 24 | Barcode No. | Формат штрих-кода № |  |  | + |  |
| 25 | Barcode2 No. | Формат доп. штрих-кода № |  |  |  | + |
| 26 | Sales Msg No. | Доп. текст № |  |  | + |  |
| 27 | SpecialPrice | Специальная цена |  |  |  | + |
| 28 | Fixed weight | Заданная масса |  |  |  | + |
| 29 | Logo No | Логотип № |  |  |  |  |
| 30 | Logo Name | Наимен. Логотипа (Ростест №) |  |  |  |  |
| 37 | Name | Наимен. 1 | + |  |  |  |
| 32 | Name2 | Наимен. 2 | + |  |  |  |
| 33 | Name3 (Rostest code) * | Код Ростеста * |  | + |  |  |
| 34 | DirectIngredient |  | + |  |  |  |

*  Rostest field for CL5000 & CL5000Jr

**  Rostest field for LP 1.6 & LP-II

As above LEVELs of PLU fields are defined as default in the ‘DATAOPTION.ini’.  You can change LEVELs on your selection.  The fields of ‘LEVEL = 9’ are disabled  in the current CL5000 version but reserved for future use.

Reference : Direct ingredient exists on the file, but level setting value is not effective.

If a direct ingredient exists, it sends that. Else, it doesn’t send that.

That is ignored when it communicates with a scale that doesn’t support a direct ingredient

.

2.5 long SendMSG()

Description: Send all Messages registered through AddMSG() to all scales.

Parameter : NONE

Return    : long, If not ‘0’, start to send.

Reference : AddMSG(), GetState()

2.6 long RecvPLU()

Description: Receive all PLUs from all connected scales to PC.

It is better practice to receive data from just one scale at a time.

Received PLUs exist in the internal buffer of the current DLL.

This function can receive data when it communicates with only one scale.

Parameter : NONE

Return    : long, If not ‘0’, start to receive.

Reference : RecvPLUData(), GetState()

2.7 long RecvMSG()

Description: Receive all MESSAGEs from all connected scales to PC.

It is better practice  to receive data from just one scale at a time.

Received MESSAGEs exist in the internal buffer of the current DLL.

This function can receive data when it communicates with only one scale.

Parameter : NONE

Return    : long, If not ‘0’, start to receive.

Reference : RecvMSGData(),GetState()

2.8 long RecvPLUData(PLUDATA)

Description: Received PLUs through RecvPLU() exist in the internal buffer of the current DLL.  Read this data one by one procedurally.

Parameter :

PLUDATA: String

PLU Data are returned according to the Table 4.1.

Return    : If ‘0’, this is the End of data. Else, this is the number of return.    Reference : RecvPLU(), GetState()

2.9 long RecvMSGData(MESSAGEDATA)

Description: Received Messages through RecvMSG() exist in the internal buffer of the current DLL.  Read this data one by one procedurally.

Parameter :

MESSAGEDATA: String

Message Data are returned according to the Table 4.2.

Return    : If ‘0’, this is the End of data. Else, this is the number of return.

Reference : RecvMSG (),GetState()

2.10 long DeletePLU(DEPARTMENTNO,PLUNO)

Description: Delete PLUs in scale.

Parameter :

DEPARTMENTNO : integer

PLUNO : integer

Return    : If not ‘0’, start to delete.

Reference : GetState()

2.11 long DeleteAllPLU(DEPARTMENTNO,PLUNO)

Description: Delete All PLUs in scale

Parameter:

DEPARTMENTNO : integer

PLUNO : integer

Return    : If not ‘0’, start to delete.

Reference : GetState()

2.12 long DeleteMSG(MESSAGENO)

Description: Delete Messages in scale.

Parameter :

MESSAGENO: integer

Return    : If not ‘0’, start to delete.

Reference : GetState()

2.13 long GetScaleInfo()

Description: Read the state information.

Read data are separated by IP and saved by the internal buffer of DLL.

Saved data should be read by GetScaleInfoData().

Parameter : NONE

Return    : If not ‘0’, read the state of a separate scale.

Reference : GetScaleInfoData()

2.14 long GetScaleInfoData(IP,INFO)

Description: Return the state information,read with GetScaleInfo(),with String value.

Parameter :

IP: String, IP of scale.

One of IPs that are already registered by Connection().

INFO: String

Message Data are returned by the Table 4.3.

Return    : integer

Info string length

Reference : GetScaleInfo ()

2.15 long GetState()

Description: 	Return the transmission state.

You can get communication state immediately after when you call up this function. But this operation occupies resource of processor, so you should not call up this function on a routine like infinite routine. If you need to call up that function spontaneously, you may as well fix the time period for calling up this.

Parameter : NONE

Return    : integer

0	  disconnect + not start

1	  connecting

11  reconnect

20  disconnection

30  send data

40  receive date

50  scale close

51  1:Success

52  2:Connect fail

53  3:Retry fail

54  4:Data error

55  5:Timeout

60  Send fail

65	  don’t looking for scale ip

70  Do not define

80  send timeout

89  Full memory

97  Data format err

99  disconnect all

Reference : SendPLU(),SendMSG(),RecvPLU(),RecvMSG(),GetScaleInfo(),

DeletePLU(),DeleteMSG(),

2.16 long GetTransStatus(IP,TYPE,STATUS)

Description: Get the version of DLL.

Parameter :

IP: String, IP of scale.

One of IPs that are already registered by Connection().

TYPE: long

STATUS: String

Return    : 0 or 1

Reference :

2.17 long GetLibVersion()

Description: Get the version of DLL.

Parameter : NONE

Return    : integer

Version Number = 100

Reference : NONE

2.18 long DisconnectAll ()

Description: Close all connected scales in network.

Parameter : NONE

Return    : integer , No meaning.

Reference : Connection()

2.19 int ReadPLU(DepartmentNo, Pluno)

Description: Call the PLU assigned.

Parameter : DepartmentNo: integer

PluNo: Integer

Return    : integer , No meaning.

Reference : Connection()

2.20 long ReadPLU(DEPARTNO,PLUNO)

Description: Upload one PLU from scale.

Parameter :

Depart No: long, Department number of PLU to be upload

PLU No: long, Number of PLU to be upload

Return    : long, No meaning

Reference : Connection()

2.21 long ReadMSG(MESSAGENO)

Description: Upload one PLU from scale.

Parameter : Message No: long

Return    : long , No meaning.

Reference : Connection()

2.22 long Stop()

Description: Disconnect all connections and stop thread..

Parameter : NONE

Return    : long, No meaning

Reference : DisconnectAll()

3. Data Structure

4.1 PLU Sending/Receiving data structure

| No. | PLU field |  | LP1.6 & LP-II | ID | Type | Length | C. type | Dec |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Name | In Russian |  |  |  |  |  |  |
| 1 | Dept no | Отдел № |  | 1 | N | 4 | ASCII | 10 |
| 2 | PLU no | Товар № |  | 2 | N | 6 | ASCII | 10 |
| 3 | PLU type | Тип товара |  | 4 | N | 2 | ASCII | 10 |
| 4 | Unit weight | Масса для цены |  | 5 | N | 2 | ASCII | 10 |
| 5 | Unit price | Цена |  | 6 | N | 10 | ASCII | 10 |
| 6 | Group no | Группа № |  | 9 | N | 4 | ASCII | 10 |
| 7 | Item code | Код товара |  | 11 | N | 13 | ASCII | 10 |
| 8 | Tare no | Тара № |  | 12 | N | 4 | ASCII | 10 |
| 9 | Tare | Масса тары |  | 13 | N | 10 | ASCII | 10 |
| 10 | Pieces | Кол-во товаров в наборе |  | 14 | N | 6 | ASCII | 10 |
| 11 | Qty unit no | Наименов. единицы № |  | 15 | N | 2 | ASCII | 10 |
| 12 | Sell by date | Годен до (дата) |  | 16 | N | 6 | ASCII | 10 |
| 13 | Sell by time | Годен до (дата) |  | 17 | N | 6 | ASCII | 10 |
| 14 | Pack date | Дата упаковки |  | 18 | N | 6 | ASCII | 10 |
| 15 | Pack time | Время упаковки |  | 19 | N | 6 | ASCII | 10 |
| 16 | Produce date | Дата производства |  | 20 | N | 6 | ASCII | 10 |
| 17 | Ingredient no = MSG | Состав продукта № |  | 25 | N | 6 | ASCII | 10 |
| 18 | Use fixed price | Фиксиров. Стоимость |  | 26 | N | 2 | ASCII | 10 |
| 19 | Traceability no | Перемещ. мясопрод. № |  | 35 | N | 6 | ASCII | 10 |
| 20 | Origin no | Страна происх. № |  | 55 | N | 6 | ASCII | 10 |
| 21 | Nutrifact | Пищевая ценность № |  | 70 | N | 4 | ASCII | 10 |
| 22 | Label no | Формат этикетки № |  | 80 | N | 4 | ASCII | 10 |
| 23 | Aux label no | Доп. формат № |  | 81 | N | 4 | ASCII | 10 |
| 24 | Barcode no | Формат штрих-кода № |  | 85 | N | 4 | ASCII | 10 |
| 25 | Barcode2 no | Формат доп. штрих-кода № |  | 86 | N | 4 | ASCII | 10 |
| 26 | Sale msg | Доп. текст № |  | 90 | N | 4 | ASCII | 10 |
| 27 | Special price | Специальная цена |  | 91 | N | 10 | ASCII | 10 |
| 28 | Fixed weight | Заданная масса |  | 100 | N | 10 | ASCII | 10 |
| 29 | Logo no | Логотип № | * |  | N | 2 | ASCII | 10 |
| 30 | Logo name | Наимен. Логотипа (Ростест №) | * |  | S | 8 | ASCII | char |
| 31 | Plu name1 | Наимен. 1 |  | 10 | S | 54 | ASCII | char |
| 32 | Plu name2 | Наимен. 2 |  | 30 | S | 54 | ASCII | char |
| 33 | name3 | Ростест № |  | 31 | S | 8(54) | ASCII | char |
| 34 | Direct ingredient |  | ? | 99 | S | 0~300 | ASCII | char |

* No. 29 & 30 are the Rostest field only for CAS LP 1.6 & LP-II Label Printing Scales

Only “O” marked items are used in CAS LP-1.6 & LP-II Label Printing Scales

Direct-Ingredient is positioned on the end of data string and should not exceed 300 Byte. In the case of CL5000JR, if that is 0, it doesn’t transmit data.

4.2 Ingredient Message structure

| Order | Name | ID | Type | Length | Code type | Dec |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Message No |  |  | 6 | ASCII | 10 |
| 2 | Message Line1 |  |  | 54 | ASCII | char |
| 3 | Message Line2 |  |  | 54 | ASCII | char |
| 4 | Message Line3 |  |  | 54 | ASCII | char |
| 5 | Message Line4 |  |  | 54 | ASCII | char |
| 6 | Message Line5 |  |  | 54 | ASCII | char |
| 7 | Message Line6 |  |  | 54 | ASCII | char |
| 8 | Message Line7 |  |  | 54 | ASCII | char |
| 9 | Message Line8 |  |  | 54 | ASCII | char |

4.3 ScaleInfo structure

| Order | Name | Type | Length | C.type | Dec |
| --- | --- | --- | --- | --- | --- |
| 1 | Current Dept no | N | 4 | ASCII | 10 |
| 2 | Current PLU no | N | 6 | ASCII | 10 |
| 3 | Tare On * | N | 1 |  |  |
| 4 | Load | N | 1 | 0,1,2 |  |
| 5 | Stable/Overload | N | 1 | 1,2,3, |  |
| 6 | Weight | N | 10 | ASCII | 10 |
| 7 | Tare | N | 10 | ASCII | 10 |
| 8 | Unit price / -00002,+00002 | N | 10 | ASCII | 10 |
| 9 | Total price/ -0002,+0002 | N | 10 | ASCII | 10 |

- Information on “Tare On”:  0: no tare,  1: tare,  2:percent tare

4. Define

//Model code

#define DF_SCALE_RELAY				1

#define DF_SCALE_15					15

#define DF_SCALE_16					16

#define DF_SCALE_16RUS					161

#define DF_SCALE_17					17

//#define DF_SCALE_20					20

#define DF_SCALE_CL5000					5000

#define DF_SCALE_CL5000JR				5010

#define DF_SCALE_LP2					200

#define DF_SCALE_LP2N					210

#define DF_SCALE_APP					20

//Action type

#define DF_ACTION_NOTHING				0

#define DF_ACTION_GETINFO				1

#define DF_ACTION_UPLOAD				2	//Upload

#define DF_ACTION_DOWNLOAD				3	//Download

#define DF_ACTION_DELETE				4	//Delete

#define DF_ACTION_DELETEALL				5	//Delete All

#define DF_ACTION_READ				6

#define DF_ACTION_WRITE				7

#define DF_ACTION_INITSCALE				8

#define DF_ACTION_RESET				9

#define DF_ACTION_SET					10

#define DF_ACTION_NORMAL				11

#define DF_ACTION_SEND					12

#define DF_ACTION_ADDCONNECTION			13

#define DF_ACTION_CONNECT				14

#define DF_ACTION_DISCONNECT				15

#define DF_ACTION_DELETECONNECTION			16

#define DF_ACTION_CONNECTALL				17

#define DF_ACTION_CONNECTONE			18

#define DF_ACTION_CANCEL				19

#define DF_ACTION_RESETCONNECTINFO			20

#define DF_ACTION_COMMAND				21

#define DF_ACTION_PING					22

#define DF_ACTION_COMPLETE				23

#define DF_ACTION_COMMSTOP				24

#define DF_ACTION_INTERPRET				25

#define DF_ACTION_DEINTERPRET			26

#define DF_ACTION_NOTIFY				27

#define DF_ACTION_SYNC					28

//Error code

#define DF_ERROR_ENDOFDATA				99

#define DF_ERROR_EXISTNOT				98

#define DF_ERROR_DATAFORMAT				97

#define DF_ERROR_UNKOWNDATA			95

#define DF_ERROR_FULLDATA				89

#define DF_ERROR_FULLDIRECT				88

#define DF_ERROR_RECEIVE				84

#define DF_ERROR_CODERANGE				82

#define DF_ERROR_CONNECTED				10

#define DF_ERROR_EMPTYDATA				7

#define DF_ERROR_ADDRESSRANGEERROR		6

#define DF_ERROR_DELETEING				16

#define DF_ERROR_RETRYOVER				79

// Receive State

#define DF_RECV_SUCCESS				1001

#define DF_RECV_FAIL					1002

// Data Type

#define DF_DATA_LOWDATA				1

#define DF_DATA_THIS					3

#define DF_DATA_STATE					4

#define DF_DATA_TEST					5

#define DF_DATA_COMMAND				6

#define DF_DATA_PING					7

#define DF_DATA_STATE_V01				4

#define DF_DATA_STATE_V02				8

#define DF_DATA_PLU_V04				218

#define DF_DATA_PLU_V03				217

#define DF_DATA_PLU_V02				9	//PLU data

#define DF_DATA_PLU_V01				10

#define DF_DATA_PLU					10

#define DF_DATA_DISC					11

#define DF_DATA_DISC_V02				216

#define DF_DATA_DEPT					12

#define DF_DATA_GROUP					13

#define DF_DATA_TAXRATE				14

#define DF_DATA_SALEMSG				15

#define DF_DATA_ORIGIN					16

#define DF_DATA_BARCODE				17

#define DF_DATA_TARE					18

#define DF_DATA_PCSUNIT				19

#define DF_DATA_INGREDIENT				20

#define DF_DATA_NUTRIFACT				21

#define DF_DATA_NUTRIFACT_V01			21

#define DF_DATA_NUTRIFACT_V02			201

#define DF_DATA_TRACE					22

#define DF_DATA_COUNTRY				23

#define DF_DATA_SLAUGHT				24

#define DF_DATA_CUT					25

#define DF_DATA_STORE					26

#define DF_DATA_CUSTOM				27

#define DF_DATA_SCROLLMSG				28

#define DF_DATA_SCROLLMSG_V01			28

#define DF_DATA_SCROLLMSG_V02			202

#define DF_DATA_SCROLLMSG_V03			203

#define DF_DATA_SECURITY				29

#define DF_DATA_REPORT				30

#define DF_DATA_LABEL					31

#define DF_DATA_IMAGE					32

#define DF_DATA_KEY					33

#define DF_DATA_PLUINFO				34

#define DF_DATA_SCALECMD				35

#define DF_DATA_DIRECTINGREDIENT			36

#define DF_DATA_LIVESTOCK				37

#define DF_DATA_LIVESTOCK_V02			204

#define DF_DATA_LIVESTOCK_V03			205

#define DF_DATA_PARAMETER				38

#define DF_DATA_SCALEINFO				40

#define DF_DATA_REPORT_CLERK				41

#define DF_DATA_REPORT_CLERK_V02			211

#define DF_DATA_REPORT_DEPT				42

#define DF_DATA_REPORT_DEPT_V02			212

#define DF_DATA_REPORT_GROUP			43

#define DF_DATA_REPORT_GROUP_V02			213

#define DF_DATA_REPORT_HOURLY			44

#define DF_DATA_REPORT_HOURLY_V02			214

#define DF_DATA_REPORT_PLU				45

#define DF_DATA_REPORT_NONPLU			46

#define DF_DATA_REPORT_REALTIME			47

#define DF_DATA_REPORT_SCALE				48

#define DF_DATA_REPORT_SCALE_V02			215

#define DF_DATA_REPORT_TAX				49

#define DF_DATA_REPORT_TOTALGRAND			50

#define DF_DATA_REPORT_TOTALPLU			51

#define DF_DATA_REPORT_TOTALNONPLU			52

#define DF_DATA_REPORT_DATETIME			53

#define DF_DATA_COUNTSYMBOL				54

#define DF_DATA_MONTHSYMBOL			55

#define DF_DATA_WEEKSYMBOL				56

#define DF_DATA_AMPMSYMBOL				57

#define DF_DATA_QUESTIONSYMBOL			58

#define DF_DATA_WEIGHTSYMBOL			59

#define DF_DATA_CURRENCYSYMBOL			60

#define DF_DATA_VERSION_TYPE1		61  // Version check CL5000JR type

#define DF_DATA_VERSION_TYPE2		62  // Version check CL5000 type

#define DF_DATA_SCANNER				63

#define DF_DATA_VERSION				64

#define DF_DATA_VERSION_TYPE3			65

#define DF_DATA_CAPTION				66

#define DF_DATA_REMAINED_PLU         		67

#define DF_DATA_REMAINED_INGREDIENT   		68

#define DF_DATA_REPORT_SALETRAN			69

#define DF_DATA_REPORT_SALETRAN_V02		70

#define DF_DATA_REPORT_SALETRAN_V03		71

#define DF_DATA_REPORT_CHANGEPRICE			72

#define DF_DATA_SYNC					73

#define DF_DATA_NOTIFY					74

#define DF_DATA_NEEDINTERPRET		    	75

#define DF_DATA_KIOSKFILE			    	76

#define DF_DATA_KEY_STANDARD_PLU			77

#define DF_DATA_KEY_STANDARD_FUNCTION		78

#define DF_DATA_KEY_POLE_PLU				79

#define DF_DATA_KEY_POLE_FUNCTION			80

#define DF_DATA_KEY_HANGING_PLU			81

#define DF_DATA_KEY_HANGING_FUNCTION		82

#define DF_DATA_KEY_SELF_PLU				83

#define DF_DATA_KEY_SELF_FUNCTION			84

#define DF_DATA_KEY_LP1_PLU				85

#define DF_DATA_KEY_LP1_FUNCTION			86

#define DF_DATA_SLAUGHTERHOUSE			87

#define DF_DATA_SPECIALUSE				88

#define DF_DATA_REPORT_SALETRAN_V04		89

#define DF_DATA_REPORT_LP17_TRAN			90

#define DF_DATA_REPORT_LP17_TRAN_ACK		91

//#define DF_DATA_REPORT_CHANGEPRICE_V02		90

#define DF_DATA_DISC_V02_WEIGHT_GLOBAL		221

#define DF_DATA_DISC_V02_COUNT_GLOBAL		222

#define DF_DATA_DISC_V02_PCS_GLOBAL			223

#define DF_DATA_DISC_PRIORITY_GLOBAL		224

#define DF_DATA_SCROLLMSG_WAITTIME_GLOBAL	225

#define DF_DATA_DATE_INFO				241

#define DF_DATA_SYSTEM_DATE				242